15DEC2015: 2HG extraction procedure for cell matrix

Samples

Castro, EX00515, N=16 cell (plates/ pellets), LOC 0000 and Stegh, EX00518, N=70 cell(plates/ pellets), LOC0000

Extraction Solvent

_____ mL of 7:2:1 (Methanol: H2O: Chloroform) +_____ µL  of 200µM 13C Gly/TCA IS ([final] 2µM),  _____µL of 100µM 13C α-KG([final]  2µM)

(0.5ml for 6cm plate, 1.5ml for 10cm plate/ .5mL to pellet/ .5mL to filter paper)?????

DATAN: diacetyl-l-tartaric anhydride- 50mg/mL in Dichloromethane-acetic acid (4:1)

Standard Mix stock (STD) and Internal Standard stock (IS)

GLY/TCA IS: 13C 100µM- need to make more

13C α-KG- need to make more?

Standard Preparation (STD Mix)--LCMS

| STD [Final-µM] | MeOH (µL) | ChCl3 (µL) | H2O (µL) | 2HG STD mix (µL) | 13C4 α-KG mix (µL) | GlyTca 13C IS mix (µL) |
| --- | --- | --- | --- | --- | --- | --- |
| STD 0 [0 µM] | 132 | 20 | 40 | 0 | 4 | 4 |
| STD 1 [0.06 µM] | 132 | 20 | 39.5 | .5 | 4 | 4 |
| STD 2 [0.125 µM] | 132 | 20 | 39 | 1 | 4 | 4 |
| STD 3 [0.625 µM] | 132 | 20 | 35 | 5 | 4 | 4 |
| STD 4 [1.25 µM] | 132 | 20 | 30 | 10 | 4 | 4 |
| STD 5 [2.5 µM] | 132 | 20 | 20 | 20 | 4 | 4 |
| STD 6 [5 µM] | 132 | 20 | 0 | 40 | 4 | 4 |
| STD A | 132 | 20 | 0 | 40 D-2HG | 4 | 4 |
| STD B | 132 | 20 | 0 | 40 L-2HG | 4 | 4 |

Sample Preparation

- Place all samples on wet ice until extraction procedure has been completed.

- While over wet ice add 0.5ml Extraction solvent containing Internal Standards (ISs) samples.

- Sonicate with probe for 5-10secs set to 4 Duty cycle and 4 output, repeat vortex.

- Keep eppendorf tubes at 4°C for 10min to allow complete extraction, remove from 4°C repeat vortex.

- Centrifuge all tubes at 14,000RPM for 10min in 4°C.

- Create a pooled sample by transferring equal volumes from each sample to an autosampler vial.

- Transfer 200µL of supernatant to an autosampler vial take to dryness using speed vac at 45° Aqueous for ~1 hr.

- Add 50µL of 50mg/mL DATAN, cap and incubate at 75°C for 30 min.

- Cool vials and dry by continuous N2 flow at RT°C for ~ 1 hr.

- Reconstitute samples in 100µL of LC grade H2O, vortex, and transfer to insert.

LC–MS Analysis

QQQ- method:

Column: Waters HSS T3 C18 50mm column.

MP A: 2mM Ammonium Formate in H2O, pH ~3.3, adjust with LC-MS grade Formic Acid

MP B: 100% ?

GCMS:  DB5MS 30x0.25x0.25 column

Notes/Observations