Metabolomic profiling reveals muscle metabolic changes following iliac arteriovenous fistula creation in mice

NMR sample preparation

Two weeks after surgery, mice were anesthetized using isoflurane and the left quadriceps muscle was rapidly dissected and immediately frozen in liquid nitrogen and stored at -80oC for metabolite extraction.

## Metabolite Extraction

Weights of frozen quadriceps specimens were weighed using a microbalance (Mettler-Toledo, Columbus, OH, USA). Next, a slightly modified FOLCH extraction was performed to extract aqueous and lipid phase metabolites. Detailed procedures for this extraction protocol can be found in our previous publications (1, 2). The aqueous phase was lyophilized overnight (Labconco Corporation, Kansas, MO, USA) and the lipid phase was dried by passing inert nitrogen gas. The resulting aqueous and lipid phase dry powders were stored at -80oC until analysis using nuclear magnetic resonance (NMR).

## NMR-based Metabolomics

The dry powder of aqueous phase samples was dissolved in 50 µL of phosphate buffer system (50 mM, pH 7.2) consisting of 0.5 mM D6-DSS, 2 mM EDTA and 0.2% NaN2. Lipid phase dry powders were dissolved in 70 µL of CDCl3 supplemented with 10 mM of pyrazine (as internal NMR standard). All samples were loaded into 1.5 mm optical density (O.D.) NMR tubes. An 800 MHz/54 mm Bruker Avance III system (Bruker BioSpin Corporation, Billerica, MA, USA) with a 5-mm TCl CryoProbe was used to acquire all 1-dimensional (1D) and 2-dimensional (2D) NMR spectra. 1D 1H spectra were acquired with the first slice of 1D nuclear Overhauser effect spectroscopy (NOESY) pulse sequence (tnnoesy) (3), utilizing parameters described previously (1, 2). 2D spectra including heteronuclear single quantum coherence (HSQC), heteronuclear multiple bond coherence (HMBC), correlated spectroscopy (COSY), and total correlated spectroscopy (TOCSY) were acquired using the standard Bruker library as well as previous work (4) to confirm metabolite identity. All spectra were acquired at room temperature.

## NMR Data Processing and Analysis

MestReNova (14.1.2-25024) software (Mestrelab Research, S.L., Santago de Compostela, Spain) was used to process all NMR spectra. For 1D NOESY spectra, line broadening of 0.22 Hz and 64k data points zero filling were applied before Fourier transformation. Before extracting peak integral areas for quantitative purpose, 1D spectra were phase and baseline (Spline method) corrected, referenced, and normalized.  All aqueous phase spectra were referenced and normalized with the D6-DSS internal NMR reference at 0.00 ppm. For lipid phase spectra, the CDCl3 peak at 7.26 ppm was used to reference all peaks and the pyrazine peak (at 8.61 ppm) was used for normalization. Muscle weight was further used to normalize peak integral areas or concentration values for all metabolites. For peaks with overlap, Chenomx Suite 8.6 NMR software (Chenomx, Inc., Edmonton, AB, Canada) was used to determine concentrations. Previously published work (1, 2, 5, 6), Chenomx Suite 8.6 NMR software, biological magnetic resonance bank (BMRB) (7), and a set of 2D spectra were used to assign and verify metabolite identity.

References:

1.	R. B. Khattri, T. Thome and T. E. Ryan: Tissue-Specific 1H-NMR Metabolomic Profiling in Mice with Adenine-Induced Chronic Kidney Disease. Metabolites, 11(1) (2021) doi:10.3390/metabo11010045

2.	R. B. Khattri, K. Kim, T. Thome, Z. R. Salyers, K. A. O'Malley, S. A. Berceli, S. T. Scali and T. E. Ryan: Unique Metabolomic Profile of Skeletal Muscle in Chronic Limb Threatening Ischemia. J Clin Med, 10(3) (2021) doi:10.3390/jcm10030548

3.	S. Ravanbakhsh, P. Liu, T. C. Bjorndahl, R. Mandal, J. R. Grant, M. Wilson, R. Eisner, I. Sinelnikov, X. Hu, C. Luchinat, R. Greiner and D. S. Wishart: Correction: Accurate, Fully-Automated NMR Spectral Profiling for Metabolomics. PLoS One, 10(7), e0132873 (2015) doi:10.1371/journal.pone.0132873

4.	C. Myer, L. Abdelrahman, S. Banerjee, R. B. Khattri, M. E. Merritt, A. K. Junk, R. K. Lee and S. K. Bhattacharya: Aqueous humor metabolite profile of pseudoexfoliation glaucoma is distinctive. Mol Omics (2020) doi:10.1039/c9mo00192a

5.	M. Krssak, L. Lindeboom, V. Schrauwen-Hinderling, L. Szczepaniak, W. Derave, J. Lundbom, D. Befroy, F. Schick, J. Machann, R. Kreis and C. Boesch: Proton magnetic resonance spectroscopy in skeletal muscle: Experts' consensus recommendations. Nmr in Biomedicine (2020) doi:10.1002/nbm.4266

6.	N. K. Srivastava, S. Pradhan, B. Mittal and G. A. Gowda: High resolution NMR based analysis of serum lipids in Duchenne muscular dystrophy patients and its possible diagnostic significance. NMR Biomed, 23(1), 13-22 (2010) doi:10.1002/nbm.1419

7.	E. L. Ulrich, H. Akutsu, J. F. Doreleijers, Y. Harano, Y. E. Ioannidis, J. Lin, M. Livny, S. Mading, D. Maziuk, Z. Miller, E. Nakatani, C. F. Schulte, D. E. Tolmie, R. Kent Wenger, H. Yao and J. L. Markley: BioMagResBank. Nucleic Acids Res, 36(Database issue), D402-8 (2008) doi:10.1093/nar/gkm957