Bile Acid Extraction

Materials:

- Gloves

- Pipettors

- Pipette tips

- Liver sample

- Ethanol

- Methanol

- Hand held homogenizer

- Probe sonicator

- Eppendorf tubes

- Autosampler vials

- Crimper

- Prepare sample:

  - Weigh out 50 mg of liver tissue or feces and place in an eppendorf tube

- Homogenization of the samples.

  - Add 1mL µl of ethanol to each sample.

  - Add 6-8 beads into each sample.

  - Homogenize the samples for about 30 seconds or until the liver is clearly disintegrated.

- Incubation of the Samples

  - Incubate the samples at 60 degrees for 30 minutes in the shaker

  - Let the samples cool to room temperature

  - Once they cool down, incubate the samples at 100 degrees for 3 minutes

    - Under this incubation the tops of the caps should be open because the ethanol will boil

- Extraction of the Bile Acids

  - After the samples are cool from the incubation, spin them at 1600 g for 10 minutes.

  - After, save the supernatant and place in a separate tube.

  - Add 500 µl of ethanol to the dry pellets and vortex for one minute

  - Centrifuge the samples at 11200 g for one minute

  - Save the supernatant and add it to the previous samples, respectively.

  - Add 500 µl of ethanol to the dry pellets for a second time and vortex again for one minute

  - Spin the samples at 11200g for one minute.

  - Combine the supernatant and add it to the other supernatants, respectively.

- Preparation for metabolomics:

  - Dry down the pooled supernatants in a drying centrifuge

  - Add 500 µl of methanol to the dried samples.

  - Vortex the samples and spin them at max speed for one minute.

  - Add 300 µl of sample to autosampler vials.