Materials

10 - 200 μL pipette (Denville)

1000 μL pipette (Denville)

1 Centrifuge (Eppendorf 5430R)

1 set of sample labeled 2 mL screw cap homogenizer tubes (VWR)

1 sets of sample labeled 1.5 mL Eppendorf tubes (Eppendorf)

250 μL autosampler vials (Thermo Fisher)

Vortexer (Any Brand)

Precellys 24 lysis and homogenization (Bertin Technologies)

Savant SPD121P SpeedVac Concentrator (Thermo Scientific)

37 °C water bath

Liquid nitrogen

1 mm Silica homogenization beads (BioSpec)

HPLC graded methanol (Sigma-Aldrich)

HPLC graded water (Sigma-Aldrich)

Chlorpropamide (Sigma-Aldrich)

Protocol Steps

- Take 600 µL of bacteria suspension after 4h incubation and centrifuge at 700 x g, 4 °C for 1 min.

- Transfer the supernatants to a new tube, centrifuge at 6000 x g, 4 °C for 3 min  and washed 3 times with PBS.

- Add 10 - 15 1 mm silica homogenization bead to the pellet, then add 1 mL of ice-cold methanol (50% v/v) containing 1 μM chlorpropamide.

- Vortex the sample briefly, then homogenize thoroughly (6500 rpm, 1 cycle, 60 s, after homogenizing for 1 min, stop for 2 min to prevent overheating).

- Freeze and thaw three times with liquid nitrogen.

- Centrifuge for 10 min, max speed, 4 °C. Transfer the supernatants to a new 1.5 mL tube.

- Dry down the samples in speedvac (takes about 3 hours).

- Resuspend the pellet in 200 μL 3% methanol.

- Centrifuge for 10 min, max speed, 4 °C.

- Transfer 150 μL of the supernatants to a 250 μL autosampler vials and store at -20°C until ready to be run.

- Pool 10 μL of each sample to a new tube for quality control. Pooled samples are prepared in duplicates.