PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 1 of 9 Table of Contents Page 1.0 BACKGROUND ........................................................................................................................2 2.0 SCOPE ........................................................................................................................................2 3.0 MATERIALS .............................................................................................................................2 3.1 Equipment ....................................................................................................................................2 3.2 Reagents .......................................................................................................................................2 4.0 PROCEDURE ............................................................................................................................3 4.1 Tissues extraction for NMR metabolomics..................................................................................3 4.2 Serum or plasma for NMR metabolomics....................................................................................4 4.3 Cecal content or feces for NMR metabolomics............................................................................5 4.4 Urine for NMR metabolomics......................................................................................................6-7 4.5 Cell extraction for NMR metabolomics........................................................................................8 5.0 REFERENCES ............................................................................................................................9

PSU METABOLOMICS CORE FACILITY SPECIFIC PROTOCOL FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 2 of 9 - BACKGROUND When the NMR-based metabonomics approaches are employed for metabolite composition analysis, the demands for extraction method will include optimal concentrations and consistent pH values for the resultant extracts so as to maximize signal-to noise ratio (SNR) and minimize the inter-sample chemical shift variations. This procedure should be used as a guide for sample preparation. Any required adjustments should be worked out during method development stage. - SCOPE This facility specific protocol (FSP) describes the tissue, serum, cecal content, feces, urine and cell sample preparation steps for analysis of metabolites via NMR. - MATERIALS 3.1 Equipment Ice Homogenization tubes Acrylic Homogenization Beads Homogenizer NMR tubes Vortexer Centrifuge 3.2 Reagents 2:1 Methanol:H2O PBS (0.045M/0.1M/1.5M, K2HPO4:NaH2PO4) D2O (deuterium oxide) TSP (trimethylsilyl propanoic acid) NaN3 EDTA-d12 KF KOH

PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 3 of 9 - PROCEDURE This procedure assumes there is adequate amount or volume of sample available to perform all the described analysis. The minimum volume of all the study samples should be checked ahead of time. If there are samples with low volume, then the volumes described in this protocol can be adjusted as long as the ratios of the reagents are kept constant. Important: Prepare a blank. Follow all steps for blank along with samples. 4.1 Tissues extraction for NMR metabolomics 1. Preparation of 0.1 M Na+/K+ PO4 Buffer (100 ml, pH = 7.4) K2HPO4: 0.8*100 (ml)*0.1(M)*174.18/1000/0.99 = 1.408 g NaH2PO4: 0.2*100 (ml)*0.1(M)*119.98/1000/0.99 = 0.242 g H2O: 100 ml*0.5 = 50 ml D2O: 100 ml*0.5 = 50 ml TSP: 0.005% (w/v)*100 = 0.005 g NaN3: 0.01 g (Preservative) 2. Tissue sample extraction - Weigh ~50 mg tissues and make a record; mark and save on ice; - Add 8-10 beads and 1 ml of pre-cooled methanol:H2O (2:1) to homogenization tubes; - Homogenize with 6500 – 1x20 – 005 program for 2-3 times; - Incubate for 5 min at room temperature; - Centrifuge at 4ºC, 11180 g for 10 min; - Transfer the supernatants to 2 ml EP tubes; - Add 0.6 ml solution (methanol: H2O (2:1)) to the pellets, repeat the above procedure (4-6 steps). - Combine the supernatants; - Dry down and then save it at -80ºC. - Resuspend in 0.6 ml 0.1 M PBS, centrifuge at 4ºC for 10 min, then transfer 0.55 ml supernatants to NMR tubes. PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 4 of 9 4.2 Serum or plasma for NMR metabolomics 1. Preparation of 0.045 M Na+/K+ PO4 Buffer (100 ml, pH = 7.4) K2HPO4: 0.8*100(ml)*0.045(M)*174.18/1000/0.99 = 0.633 g NaH2PO4: 0.2*100(ml)*0.045(M)*119.98/1000/0.99 = 0.110 g NaCl: 0.9 g H2O: 50ml D2O: 50 ml 2. Serum or plasma sample extraction If the volume of serum is enough: - 200 µl samples mixed with 400 µl PBS (0.045M) containing 50% D2O; - Vortex samples for 10 s; - Centrifuge at 18100 g for 10 minutes, 4ºC; - Transfer 550 µl supernatants into 5.0 mm NMR tubes. If the volume of serum is not enough (≤ 50 µl): - 20 µl samples mixed with 40 µl PBS (0.045M) containing 100% D2O; - Vortex samples for 10 s; - Centrifuge at 18100 g for 10 minutes, 4ºC; - Transfer 60 µl supernatants into 1.7 mm NMR tubes. (NS = 256 when acquire 1H NMR spectra) PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 5 of 9 4.3 Cecal content or feces for NMR metabolomics 1. Preparation of 0.1 M Na+/K+ PO4 Buffer (100 ml, pH = 7.4) K2HPO4: 0.8*100 (ml)*0.1(M)*174.18/1000/0.99 = 1.408 g NaH2PO4: 0.2*100 (ml)*0.1(M)*119.98/1000/0.99 = 0.242 g H2O: 100 ml*0.5 = 50 ml D2O: 100 ml*0.5 = 50 ml TSP: 0.005% (w/v)*100 = 0.005 g NaN3: 0.0195 g (Preservative) 2. Cecal content or feces sample extraction - Weigh 50~60 mg samples and make a record; mark and save it on ice; - Add 8-10 beads and 0.6 ml PBS (0.1M) solution containing 50% D2O to homogenization tubes, Vortex 30s; - Homogenization with 6500 – 1x20 – 005 program for 2-3 times; - Freeze-thawing two times with Liquid nitrogen; - Centrifuge at 4ºC, 11180 g for 10 min; - Transfer the supernatants to 2 ml new EP tubes; - Add 0.6 ml PBS solution to the pellets followed with Vortex 30s and Centrifuge at 4ºC, 11180 g for 10 min. (no dry down step) - Combine the supernatants, centrifuge at 4ºC and 16099 g for 10 min; Transfer the supernatants (0.55 ml) to NMR tubes. PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 6 of 9 4.4 Urine for NMR metabolomics 1. Preparation of 1.5 M Na+/K+ PO4 Buffer (100 ml, pH = 7.4) K2HPO4: 0.8*100(ml)*1.5(M)*174.18/1000/0.99 = 21.113 g NaH2PO4: 0.2*100(ml)*1.5(M)*119.98/1000/0.99 = 3.636 g H2O: 100 ml*0.5 = 50 ml D2O: 100 ml*0.5 = 50 ml TSP: 0.005% (w/v)*100 = 0.005 g NaN3: 0.01 g (Preservative) 2. Preparation of 5M KF solution (25 ml, save in a plastic container) KF·2H2O: 25(ml)*5(M)*94.13/1000/0.99 = 11.885 g H2O: 25 ml 3. Preparation of 0.12M EDTA-d12 solution (25 ml) EDTA-d12: 25(ml)*0.12(M)*304.34/1000 = 0.913 g KOH: 0.593 g H2O: 25 ml 4. Urine sample extraction For mice urine - 60 µl urine mixed with 4.8 µl KF (5M) and 440 µl D2O (20%); - Vortex samples for 10 s; - Centrifuge at 11800 g for 10 minutes, 4ºC; - Add 5 µl EDTA-d12 (0.12M) into NMR tubes; - Transfer 450 µl supernatants into NMR tubes, blending; - Add 45 µl PBS (1.5M) into NMR tubes. For human urine - 500 µl urine mixed with 14 µl KF (5M); - Vortex samples for 10 s; - Centrifuge at 11800 g for 10 minutes, 4ºC; - Add 8.3 µl EDTA-d12 (0.12M) into NMR tubes; - Transfer 450 µl supernatants into NMR tubes, blending; - Add 45 µl PBS (1.5M) into NMR tubes. PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 8 of 9 4.5 Cell extraction for NMR metabolomics 1. Preparation of 0.1 M Na+/K+ PO4 Buffer (100 ml, pH = 7.4) K2HPO4: 0.8*100 (ml)*0.1(M)*174.18/1000/0.99 = 1.408 g NaH2PO4: 0.2*100 (ml)*0.1(M)*119.98/1000/0.99 = 0.242 g H2O: 100 ml*0.5 = 50 ml D2O: 100 ml*0.5 = 50 ml TSP: 0.005% (w/v)*100 = 0.005 g NaN3: 0.01 g (Preservative) 2. Cell sample extraction - Add 1 ml of pre-cooled methanol:H2O (2:1); - Vortex samples for 10 s; - Freeze-thawing three times with Liquid nitrogen; - Centrifuge at 11800 g for 10 minutes, 4ºC; - Transfer the supernatants to 2 ml new EP tubes; - Add 0.6 ml solution (methanol: H2O (2:1)) to the pellets, repeat the above procedure (2-5 steps). - Combine the supernatants; - Dry down and then save it at -80ºC. - Resuspend in 0.6 ml 0.1 M PBS (D2O), centrifuge at 4ºC for 10 min, then transfer 0.55 ml supernatants to NMR tubes.

PSU METABOLOMICS CORE FACILITY SPECIFIC PROCEDURE FSP Number: PSU- HILSMCF-NMR-001 Version number: 01 FSP Title: NMR Metabolomics PSU Huck Institutes of the Life Sciences Metabolomics Core Facility at University Park Page 9 of 9 5.0 REFERENCES Junfang Wu, et al. “An optimised sample preparation method for NMR-based faecal metabonomic analysis” Analyst, 2010 May;135(5):1023-30. doi: 10.1039/b927543f Limiao Jiang, et al. “Eliminating the dication-induced intersample chemical-shift variations for NMR-based biofluid metabonomic analysis” Analyst, 2012 Sep 21;137(18):4209-19. doi: 10.1039/c2an35392j