# Cold adaptation shapes the robustness of metabolic networks in Drosophila melanogaster.

Williams CM1§*, Watanabe M2, Guarracino MR3, Ferraro MB3†, Edison AS1, Morgan TJ4, Boroujerdi AFB2, Hahn DA1

1University of Florida, Gainesville, FL, USA

2Claflin University, Orangeburg, SC, USA

3National Research Council of Italy, Naples, Italy

4Kansas State University, Manhattan, KS, USA

§Present address: University of California, Berkeley, CA, USA

† Present address: University of Rome, La Sapienza, Italy

*Corresponding author: cmw@berkeley.edu

Running title: Metabolic homeostasis in cold-adapted Drosophila

Methanol/chloroform extraction

- Weigh flies directly into vial with 0.7mm ceramic beads in preweighed and labeled 2 mL bead beating tube

- Weigh to get wet weight of flies

- Freeze, lyophilize overnight, weigh to get dry metabolites

- Prepare glass test tubes with nonpolar solvents – on ice (1mL chloroform, 0.5 mL water)

- Homogenize dried flies for 20s in bead beater (speed 4)

- Add polar solvents (1mL methanol, 0.4 mL water) to lyophilized flies in bead beating tube

- Homogenize dried flies for 20s in bead beater (speed 5)

- Add polar solvent/fly mixture to nonpolar solvents in glass test tube

- Vortex for 1min

- Leave on ice for 10 min

- Centrifuge at 2000xg at 4˚C for 5min

- Remove upper polar phase and place in second set of preweighed and labeled microcentrifuge tube

- (Freeze at -20˚C if not ready to centrivap)

- Centrivap until dry – weigh to get dry metabolites

- Rehydrate with 620 μl 0.1M sodium phosphate buffer (pH 7.3)

- Vortex for 30s

- Centrifuge at 10000rpm for 5min at 4˚C

- Transfer 600 μl to labeled NMR tube